Lesson Plan Template

# Location of Course: __KCFR___________		Date: _____2008___

Instructor: ________________________		Length of Class: ___30 min

Target Audience: ______Suppression personnel________

Lesson Title: _______PPE Inspection-Structural Gear

Objectives:

Review KCf&R/SOP section 12-03-0 Protective Clothing Inspect other FF’s PPE Learn what your partner carries in their turnouts Learn why they carry what they carry Evaluate if you need to carry more tools on yourself or get rid of some of the items you currently have

Material:

All structural fire PPE All personnel

| Clock Time: | Main Points: | Methods, Media: |
| --- | --- | --- |
| 30 Min | Review KCf&R/SOP section 12-03-0 Protective Clothing With a partner inspect each others PPE in accordance with NFPA 1851 Chapter 6 Individual members shall conduct a routine inspection of their protective ensembles and ensemble elements after each use. Coat and trouser garment elements shall be inspected for the following: -soiling -contamination -physical damage such as rips and cuts -thermal damage (chars, burns, significant discoloration) -loss of seam integrity -damaged or missing reflective trim Helmet shall be inspected for: -cracks, dents, and abrasions -thermal damage -check to make sure earflap is intact -face shield and/or goggles intact, cracked, or broken -damaged or missing reflective trim Inspect Gloves for: -Soiling -Physical damage -Correct size (shrinkage) -intact seams Inspect Footwear for: -Soiling -Physical damage -Exposed or deformed steel toe, midsole or shank -loss of water resistance |  |
|  | Don PPE to evaluate: -correct sized PPE -make sure you are not walking on trouser leg bottoms |  |
|  | If you find any problems with PPE notify your shift officer. Shift Officer, notify the PPE suppression equipment coordinator with your crews’ problems. Follow the chain of command. |  |
|  | What’s in your pockets? Go to this link for more information http://www.vententersearch.com/pockets.htm After the PPE inspection have everyone one at a time: -show what they have in their turnout pockets -if it is something extraordinary, explain the significance of the tool -evaluate the possible need for more tools to put in your turnouts (i.e., hose strap, crescent wrench, valve stem remove, door chock, etc) This drill is used to be a constructive learning tool for each other. For example if you know your partner has a pair of wire cutters, entanglement occurs inside a collapsed structure, and they are unconscious, you can use their tool to cut wires. Extra items, besides regular PPE, recommended to carry in turnouts: -flash light(s) (small, cheap, disposable) -knife(s) -hose strap(s) -wire cutter -pliers -door chock -crescent wrench -extrication gloves -personnel rope bag -anything else that can be useful (see link) |  |